Individual assignment 1 – IoT fundamentals

Coverage: Week 1 -5

Name:

Student ID:

Section A – Multiple choice (20 marks – 10 questions x 2 marks)

- Which statement best describes a complete IoT system?

A.  A sensor that measures temperature

B.  A microcontroller running a Python program

C.  A system that senses, communicates, processes data, and supports an application

D.  Any device connected to Wi-Fi

Answer: C

- Which device primarily converts a physical quantity into measurable data?

A.  Actuator

B.  Sensor

C.  Router

D.  MQTT broker

Answer: B

- A sensor records one temperature value every 30 seconds.

A.  network traffic

B.  time-series data

C.  an IP address

D.  an MQTT topic

Answer: B

- Which is an example of a sensor-data quality problem?

A.  Missing measurement

B.  IPv6 address

C.  Wi-Fi connection

D.  MQTT subscription

Answer: A

- What is the main purpose of serialization?

A.  To increase wireless range

B.  To convert structured data into a representation suitable for transmission or storage

C.  To select an IP address

D.  To increase sensor accuracy

Answer: B

- A fitness sensor sends a small amount of data to a nearby smartphone and must operate from a small battery. Which connectivity technology is most appropriate?

A.  Ethernet

B.  BLE

C.  Satellite

D.  LoRa for every case

Answer: B

- Which statement about IEEE 802.15.4 and Zigbee is correct?

A.  IEEE 802.15.4 is built on Zigbee

B.  They are exactly the same technology

C.  Zigbee can use IEEE 802.15.4 as its low-power PHY/MAC foundation

D.  Zigbee is a transport protocol such as TCP

Answer: C

- Which transport protocol provides a connection-oriented, reliable, ordered byte stream?

A.  IP

B.  UDP

C.  TCP

D.  MQTT

Answer: C

- Which application protocol is based on publish–subscribe communication?

A.  HTTP

B.  MQTT

C.  IPv6

D.  Ethernet

Answer: B

- An MQTT alarm message is important and should be delivered, but receiving the same alarm twice is acceptable.

A.  QoS 0

B.  QoS 1

C.  QoS 2 is always required

D.  MQTT has no delivery levels

Answer: B

Section B – Short Answer (30 marks – 5 questions x 6 marks)

- IoT Device vs IoT System.

A smartwatch measures a user's heart rate and displays the measurement on its own screen. Explain the difference between:

1. an IoT device, and

2. a complete IoT system.

Use the smartwatch example in your explanation

An IoT device is one thing with a sensor, like the smartwatch itself. It measures heart rate and shows it on its screen.

A complete IoT system is more than the device. The watch would also send the data over Bluetooth or Wi-Fi to a phone or cloud, where it is stored, processed and shown in an app (for example heart rate history or alerts).

So the watch showing the value on its own screen is only a device. It becomes an IoT system when it senses, communicates, processes data and supports an application.

12. Sensor data

A room-temperature sensor produces:

10:00   22.5 °C

10:01   22.7 °C

10:02   missing

10:03   22.8 °C

10:04   22.9 °C

Answer:

a. What type of data is this?

Time-series data, because each value has a timestamp and they come in order.

b. Identify one data-quality problem.

The value at 10:02 is missing.

c. Explain one possible way an application could handle the problem.

The app could estimate it from the values next to it, for example the average of 22.7 and 22.8 which is about 22.75 °C. Or it can skip that value and mark it as missing.

13. Data Representation

sensor_data = {

"device": "room01",

"temperature": 22.8

}

And:

{"device": "room01", "temperature": 22.8}

Explain:

a. What is the first representation?

A Python dictionary. It is a data structure that lives in the memory of the program.

b. What is the second representation?

A JSON string (text). It is the serialized form of the same data.

c. Why might serialization be required before transmitting the information?

A network can only send bytes, not Python objects. So the dictionary must be turned into text/bytes like JSON. Then the receiver, which may use another language, can read it back.

14. TCP and UDP

Explain two differences between TCP and UDP.

Your answer should refer to:

- connection establishment;

- delivery behavior.

Do not simply state that one is "faster."

Connection: TCP is connection-oriented, it sets up a connection first with a three-way handshake before sending data. UDP is connectionless, it just sends datagrams without any setup.

Delivery: TCP is reliable, it uses acknowledgements and resends lost data, and the data arrives in order. UDP gives no guarantee, packets can be lost, duplicated or arrive out of order and UDP does not fix it.

15. Request–Response vs Publish–Subscribe

Explain the difference between:

1. request–response; and

2. publish–subscribe.

Give one suitable IoT example for each.

Request–response: a client asks a server and the server answers that client directly. Example: a phone app sends an HTTP request to a smart thermostat to get the current temperature.

Publish–subscribe: a sender publishes messages to a topic on a broker, and everyone subscribed to that topic gets them. The publisher and subscriber do not know each other. Example: a temperature sensor publishes to home/room1/temperature with MQTT and a dashboard subscribes to it.

Section C – Applied Scenario Questions (40 marks – 2 questions x 20 marks)

16. Smart Greenhouse IoT System

A greenhouse contains battery-powered soil-moisture sensors distributed across a large farm.

Each sensor:

- sends a small measurement every 15 minutes;

- may be located several kilometres from the farm office;

- must operate for a long time using battery power;

- does not need to send images or video.

a. Identify three communication requirements that are important when selecting connectivity for these sensors. (3 marks)

1. Long range (several kilometres).

2. Low power use, so the battery lasts a long time.

3. Low data rate is enough, only small messages every 15 minutes.

b. Choose a suitable connectivity technology.

Possible examples include:

- Wi-Fi

- BLE

- Ethernet

- LoRa

- cellular IoT

Give two reasons for your choice. (5 marks)

LoRa.

1. It has long range, a few kilometres, so it can reach the farm office.

2. It uses very little power and the small messages fit its low data rate, so the batteries last a long time.

c. The gateway receives:

{

"sensor": "soil07",

"moisture": 41.2

}

What is JSON providing in this system? (3 marks)

JSON gives a common data format. It puts the reading into structured text with names and values, so the gateway and the other applications can read and understand it the same way.

d. Suppose many greenhouse sensors continuously report measurements to:

- a dashboard;

- a database;

- an irrigation-control application.

Would request–response or publish–subscribe be more suitable?

Explain. (4 marks)

Publish–subscribe is more suitable. The sensors send data all the time and three different applications need the same data. Each sensor publishes once and the dashboard, database and irrigation application all subscribe. The sensors do not need to know who the receivers are, and new receivers can be added easily.

e. Name a suitable application protocol for the communication model selected in part (d).

Explain the role of the broker if your selected protocol uses one. (5 marks)

MQTT.

The broker is the server in the middle. It receives the messages published by the sensors and forwards them to all clients subscribed to that topic. So publishers and subscribers never connect to each other directly, only to the broker.

17. Campus Environmental Sensor

A university installs battery-powered environmental sensors inside buildings.

Each sensor:

- has limited memory and processing capability;

- sends small temperature and humidity messages;

- uses IEEE 802.15.4;

- should participate in an IPv6-based IoT network;

- is occasionally queried by a local application.

a. Which application protocol is suitable for a constrained request–response device? (2 marks)

CoAP (Constrained Application Protocol).

b. Which transport protocol is commonly used with this application protocol? (2 marks)

UDP.

c. What technology can adapt IPv6 communication to an IEEE 802.15.4 constrained network? (3 marks)

6LoWPAN. It compresses the IPv6 headers and fragments packets so they fit into the small IEEE 802.15.4 frames.

d. Complete the protocol stack: (5 marks)

Environmental Application

↓

CoAP

↓

UDP

↓

IPv6

↓

6LoWPAN

↓

IEEE 802.15.4

e. Explain the role of each of the following: (8 marks)

- application protocol;

- transport protocol;

Application protocol (CoAP): defines the format and meaning of the messages between the applications, for example a GET request to read the temperature and the response with the value.

Transport protocol (UDP): delivers the data between the right applications on the two devices using port numbers. UDP does this with little overhead and no connection, which is good for constrained devices.

Section D — Integrated IoT Communication (10 marks)

18 — Trace the Data

Consider the following IoT system:

Temperature Sensor

↓

Python application

↓

JSON

↓

Wi-Fi

↓

IP

↓

TCP

↓

MQTT

↓

Broker

↓

Monitoring Application

Explain the role of each of the following:

| Technology | Role |
| --- | --- |
| JSON | Data format. Turns the temperature reading from the Python program into structured text that can be sent and understood by the other side. |
| Wi-Fi | Wireless link. Carries the data from the device to the access point / local network. |
| IP | Addressing and routing. Gets the packets from the device to the IP address of the broker across networks. |
| TCP | Reliable transport. Makes a connection to the broker and makes sure the data arrives complete and in order. |
| MQTT | Application protocol. The device publishes the reading to a topic on the broker, and the monitoring application subscribes to that topic to receive it. |